Skill 11-1

Evaluation Checklist

Objective 4:	Dam a stream with a ladder and salvage cover. [NFPA® 1002, 5.2.1, 5.2.2]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will dam a stream with a ladder and salvage cover. A minimum of two firefighters is required to perform this skill. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Appropriate PPE - A ladder (preferably a straight ladder) long enough to span waterway bank to bank - Straight pry bar, rod or post | - Stream suitable for damming - Safety equipment as required by local SOPs - Salvage cover(s) |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 4:	Dam a stream with a ladder and salvage cover. [NFPA® 1002, 5.2.1, 5.2.2]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Spread the salvage cover flat on the ground. If the ladder is longer than the salvage cover, two salvage covers may be used. |  |  |
| 2. | Lay the ladder down on one of the long sides of the salvage cover(s). |  |  |
| 3. | Grab the salvage cover and begin to roll the ladder with the salvage cover, wrapping the beams. |  |  |
| 4. | Roll the ladder until there is approximately 4 feet (1 m) of cover remaining on the ground that is free to form a flap. |  |  |
| 5. | Place the ladder wrapped with a salvage cover to an area where the stream or ground is flat. NOTE: This will allow the most contact with the bottom of the ladder. The ladder may need to be angled in the stream to make contact with the sides of the banks. |  |  |
| 6. | Stretch the free flap upstream, and secure it to the ground. NOTE: The more secure this section is, the less water will escape under the dam. |  |  |
| 7. | Tuck in the ends of the salvage cover and adjust as necessary to limit excessive leakage. |  |  |
| 8. | Support the middle of the down stream side of the ladder with a straight pry bar, rod, or post as the weight of the water may push the dam over. If using as a water diversion, support the ladder on both ends. |  |  |