Skill 4-5

Evaluation Checklist

Objective 8:	Position pumper and make connections for a tandem pumping operation. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will position pumper and make connections for a tandem pumping operation. Students must work as part of a crew for this skill sheet. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs) when performing this skill.

Resources

| - Appropriate PPE - Two pumping apparatus - Manufacturer’s operator’s manual - Large diameter supply hose - Two 2½-inch (65 mm) or 3-inch (77 mm) hoselines | - Bell reducer or Siamese fitting - Spanner and hydrant wrenches - Rubber mallet - Adapters as needed - Wheel chocks - Portable or handheld radio |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 8:	Position pumper and make connections for a tandem pumping operation. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

NOTE: The instructor will provide students with information on positioning the pumper. The instructor may choose to provide details of an incident scenario.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| Attack Pumper |  |  |  |
| 1. | Position at the scene according to the needs of the incident. |  |  |
| 2. | Supply attack lines, elevated master streams, or FDC. Complete hookups per local SOPs. |  |  |
| 3. | After the supply pumper has reverse laid toward the hydrant, connect the supply hose to the appropriate intake. |  |  |
| 4. | Switch from onboard to external water supply when supply hoses are charged. |  |  |
| Supply Pumper |  |  |  |
| 5. | Reverse lay supply hose from the attack pumper to the hydrant. |  |  |
| 6. | Put the pump in gear and prepare to flow water. |  |  |
| 7. | Establish a water supply. |  |  |
| 8. | Connect supply hose to the appropriate pump discharge. |  |  |
| 9. | Discharge water through the supply hose to the attack pumper. |  |  |
| 10. | Coordinate pressure with the attack pumper driver/operator. |  |  |