Skill 4-1

Evaluation Checklist

Objective 4:	Position pumper and make large diameter intake hose connections. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will position pumper and make large diameter intake hose connections. Student must work as part of a crew for this skill. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Appropriate PPE - Pumping apparatus - Large diameter supply hose - Spanner and hydrant wrenches - Wheel chocks | - Rubber mallet - Two 2½” (65 mm) gate valves as required - Adapters as required - Chafing blocks, if required |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 4:	Position pumper and make large diameter intake hose connections. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

NOTE: This skill sheet provides general steps for positioning a pumper to make large diameter intake hose connections. Always follow the SOPs of the jurisdiction.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| Side Intake |  |  |  |
| 1. | Position apparatus a few feet (meters) short of hydrant. Chock wheels. |  |  |
| 2. | Connect LDH to hydrant and lay hose to street, folding approx 3 feet (1 m) back onto itself (place two full counterclockwise twists in the hose if sexless couplings are not used). |  |  |
| 3. | Connect LDH to intake. |  |  |
| 4. | Charge LDH and bleed air if required. |  |  |
| 5. | Place apparatus into pump gear. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| Front or Rear Intake |  |  |  |
| 1. | Position apparatus a few feet (meters) short of hydrant (for front intake) or a few feet (meters) beyond the hydrant (for rear intake).; |  |  |
| 2. | Connect LDH to hydrant (place two full counterclockwise twists in the hose if sexless couplings are not used). Connect 2 ½ inch (65 mm) gate valves to unused outlets on a dry barrel hydrant, in case more water is needed. |  |  |
| 3. | Connect LDH to intake. |  |  |
| 4. | Charge LDH. |  |  |
| 5. | Place apparatus into pump gear. |  |  |