Skill 4-3

Evaluation Checklist

Objective 6:	Position pumper and make multiple intake connections from a hydrant. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will position pumper and make multiple intake connections. Students must work as part of a crew for this skill sheet. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Appropriate PPE - Pumping apparatus - Manufacturer’s operator’s manual - Large diameter supply hose - Spanner and hydrant wrenches - Rubber mallet - Wheel chocks | - Two 2½-inch (65 mm) or 3-inch (77 mm) hoseline(s) - Two 2½-inch (65 mm) gated hydrant valves - Adapters as needed |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 6:	Position pumper and make multiple intake connections from a hydrant. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

Note: This skill sheet provides general steps for positioning a pumper and making multiple intake connections. Always follow the SOPs of the jurisdiction.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Properly position apparatus in relation to fire hydrant. Chock wheels. |  |  |
| 2. | Connect LDH and a 2½-inch (65 mm) gate valve to the hydrant. Lay hose to street, folding approx 3 feet (1 m) back onto itself (place two full counterclockwise twists in the hose if sexless couplings are not used). |  |  |
| 3. | Connect LDH to intake. |  |  |
| 4. | Charge LDH and bleed air. |  |  |
| 5. | Connect additional 2½- or 3-inch (65 or 77 mm) lines to gate valve and apparatus. |  |  |
| 6. | Charge additional hoselines. |  |  |
| 7. | Place apparatus into pump gear. |  |  |