Skill 4-2

Evaluation Checklist

Objective 5:	Position pumper and connect to 2½-inch (65 mm) hydrant outlets.

[NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will position pumper and connect to 2½-inch (65 mm) hydrant outlets. Students must work as part of a crew for this skill sheet. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Appropriate PPE - Pumping apparatus - Manufacturer’s operator’s manual - One or two 2½-inch (65 mm) or 3-inch (77 mm) hoseline(s) | - Adapters as needed - Two 2½-inch (65 mm) gated hydrant valves as required - Hydrant and spanner wrenches - Wheel chocks |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 5:	Position pumper and connect to 2½-inch (65 mm) hydrant outlets. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Position apparatus as close to the hydrant as possible. Chock wheels. |  |  |
| 2. | Lay out needed amount of 2½- or 3-inch (65 or 77 mm) hose, as required. |  |  |
| 3. | Attach 2½-inch (65 mm) gate valves to hydrant. |  |  |
| 4. | Connect 2½- or 3-inch (65 or 77 mm) hoselines to the gate valves. |  |  |
| 5. | Connect 2½- or 3-inch (65 or 77 mm) hoselines to the apparatus intake. |  |  |
| 6. | Charge supply lines, bleeding air as necessary. |  |  |