Skill 14-1

Evaluation Checklist

Objective 10:	Install and operate an in-line foam eductor. [NFPA® 1002, 5.2.3]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will install an in-line foam eductor. This skill requires firefighters to work as a crew. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Appropriate PPE - Pumping apparatus - Foam eductor and nozzle with matched gallons per minute (GPM) setting | - Foam concentrate - Water supply - Hoselines and other related tools and equipment |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 10:	Install and operate an in-line foam eductor. [NFPA® 1002, 5.2.3]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Select the proper foam and percentage of application for the fuel type. |  |  |
| 2. | Check the nozzle and eductor, and ensure they are hydraulically compatible. |  |  |
| 3. | Check and set the eductor for the percentage concentration needed. |  |  |
| 4. | Attach the eductor to the discharge outlet or in the hose line. |  |  |
| 5. | Connect the nozzle to the discharge end of the hose. Check with the manufacturer’s instructions to ensure the length of hose does not exceed recommendations. |  |  |
| 6. | Place the eductor pick-up tube into the foam concentrate. Ensure the tube is not more than 6 feet (2 m) below the eductor. |  |  |
| 7. | Increase the water pressure in accordance with manufacturer’s instructions and local SOPs. |  |  |
| 8. | Ensure the nozzle operator has sufficient foam in accordance with manufacturer’s instructions and local SOPs. If the foam line is not operating properly, refer to the troubleshooting guide. |  |  |
| 9. | Once the foam evolutions are complete, make sure all appliances are thoroughly flushed with clean water. |  |  |