Skill 10-4

Evaluation Checklist

Objective 10:	Make the transition from the apparatus water tank to an external pressurized water supply.  [NFPA® 1002, 5.2.1]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will make the transition from the apparatus water tank to an external water supply. This skill may require additional personnel to help the student perform this skill. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Appropriate PPE - Pumping apparatus - Apparatus operator’s manual - External pressurized water supply (hydrant or supply hose from another fire pump) | - Wheel chocks - Additional personnel if applicable - Hoselines, nozzles, and other related tools and equipment - Spanner and hydrant wrenches |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 10:	Make the transition from the apparatus water tank to an external pressurized water supply.  [NFPA® 1002, 5.2.1]

NOTE:  Always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Position the apparatus in a safe position, and immobilize it by setting the parking brake and chocking the wheels. |  |  |
| 2. | Engage the pump. |  |  |
| 3. | Open the tank-to-pump valve. |  |  |
| 4. | Set the transfer valve to the SERIES (PRESSURE) position (if necessary). |  |  |
| 5. | Open discharge(s) and increase the throttle setting to obtain the desired pressure, priming (if necessary). |  |  |
| 6. | Set the relief valve or pressure governor. |  |  |
| 7. | When an external supply becomes available, open the intake valve while adjusting the throttle to avoid a pressure surge. - If using a pressure governor, switch to “rpm” mode and ensure air is bled from the supply line before opening intake and switch back to “pressure” mode. - Adjust throttle to maintain discharge pressure. - Position tank-to-pump valve per local SOPs. |  |  |
| 8. | As soon as an adequate supply of water is available, divert enough of it through the tank fill line to replenish the supply in the tank |  |  |