Skill 10-6

Evaluation Checklist

Objective 12:	Draft from a static water supply. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will position pumper and draft from a static water supply. Students must work as part of a crew for this skill sheet. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Appropriate PPE - Additional personnel - Pumping apparatus - Manufacturer’s operators manual - Pumping apparatus/primer pump - Static water supply - Hard intake hose | - Strainer - Length of rope for typing up strainer - Wheel chocks - Rubber mallet - Roof ladder (optional) - Personal flotation devices (PFD) |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 12:	Draft from a static water supply. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Select the drafting site, ensuring that there is an adequate water depth and water quality and that the location is accessible and stable. |  |  |
| 2. | Ensure apparatus will operate on an improved surface. |  |  |
| 3. | Position the apparatus to allow connection of hard intake hose. CAUTION: Any personnel working near the edge of bodies of water are required to wear a personal flotation device (PFD). |  |  |
| 4. | Assemble hose and strainers. |  |  |
| 5. | Connect the sections of intake hose. - Align each section of hose before turning it. - Hand tighten connections. c. Use a rubber mallet as necessary to make connections airtight. |  |  |
| 6. | Connect the strainer to the end of the intake hose. |  |  |
| 7. | Tie the guide rope to the strainer securely. |  |  |
| 8. | Connect intake hose to apparatus, ensuring all connections are tight. |  |  |
| 9. | Place the intake hose and strainer into the water slowly to avoid stirring up the sediment and dirt in the water. |  |  |
| 10. | Place pump in gear. Operate primer until waterflow is established (operate no longer than manufacturer recommendations). |  |  |
| 11. | Operate pump/pressure governor in correct mode. |  |  |