Skill 10-2

Evaluation Checklist

Objective 8:	Engage and disengage a pump. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will engage and disengage a pump. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Pumping operator’s manual - Wheel chocks if applicable | - Appropriate PPE - Pumping apparatus |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 8:	Engage and disengage a pump. [NFPA® 1002, 5.2.1, 5.2.2, 5.2.4]

Note:  The information contained in this skill sheet provides sound, generic advice for engaging and disengaging a PTO for an apparatus with an automatic transmission. Follow manufacturer’s instructions for apparatus with a manual transmission.

While many of the basic principles are the same for all makes and models of pumps and apparatus, each manufacturer’s equipment has special nuances with which the driver/operator must be familiar. For that reason, the driver/operator should always consult the operator’s manual for the apparatus and pump being operated. Remember to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Engaging the Pump |  |  |
| 1. | Bring the apparatus to a full stop. |  |  |
| 2. | Shift the transmission into neutral. |  |  |
| 3. | Set the apparatus parking brake. |  |  |
| 4. | Operate the pump shift control to transfer power from the drive axle to the pump drive. NOTE: If the driver/operator must leave the cab to perform this step, he or she should secure the vehicle with wheel chocks prior to engaging the pump. |  |  |
| 5. | Shift the transmission into the appropriate gear for pumping. NOTE: A pump indicator light should illuminate to signify to the driver/operator that the apparatus is ready to pump. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Disengaging the Pump |  |  |
| 1. | Use the throttle control to reduce the engine speed to idle. |  |  |
| 2. | Shift the transmission into neutral. NOTE: The pump indicator light should no longer be illuminated. If it is still lit, repeat the procedure. |  |  |
| 3. | Operate the pump shift control to transfer power from the pump drive to the drive axle. |  |  |