Skill 13-4

Evaluation Checklist

Objective 9:	Establish, operate, and shut down a multiple portable tank water shuttle dump site. [NFPA® 1002, 10.2.2, 10.2.3]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will establish a water shuttle dump site. Students must work as part of a crew for this skill sheet. This skill requires one dump site pumper and at least two shuttle tenders and driver/operators with crews. The most efficient method for operating a dump site is to have tenders dump their loads into one or more portable water tanks. A dump site pumper is responsible for drafting from the one or more portable tanks and supplying water to the attack pumper. Most jurisdictions find this to be the optimum way to provide a constant high flow of water using a shuttle operation. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Appropriate PPE - One dump site pumper with hard intake hose with and low-level strainer - At least two shuttle tenders - Water supply officer - 2 or more apparatus-mounted portable water tanks | - One salvage cover for each portable tank to be used - Water tank manufacturer’s setup instructions - Length of utility rope for suspending siphon hose - Jet-siphon |
| --- | --- |

NOTE: This skill sheet is written for a jet siphon. A plain siphon, commercial tank-connecting device, permanent tank gravity drain, or drain tunnel connector may also be used.

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 9:	Establish, operate, and shut down a multiple portable tank water shuttle dump site. [NFPA® 1002, 10.2.2, 10.2.3]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Set up the first portable tank in the same manner as described in Skill Sheet 13-3. Make sure that the tank drain is on the low side of the tank. |  |  |
| 2. | Dump site pumper driver/operator and crew: Deploy hard intake hose and strainer into the first portable tank. |  |  |
| 3. | First tender on the scene: Dump water into the first tank. The dump site pumper may then begin the drafting operation. |  |  |
| 4. | First tender on the scene: Set up a second portable tank next to the first one. A tip-to-tip diamond-shaped arrangement is generally the preferred method for arranging the tanks. |  |  |
| 5. | First tender on the scene: Assemble the jet siphon equipment. Place the end with the siphon in the second portable tank and the discharge end of the assembly over the edge of the first portable tank. |  |  |
| 6. | Next water tender: Dump a load of water into the second portable tank. Once the water level is sufficient, charge the jet siphon supply hose to begin the process of transferring water from the second tank to the first. |  |  |
| 7. | Next water tender: If additional portable tanks are used, each should be set up and operated as described in steps 4 through 6. Tenders should always dump in the end-most tank that has room for water. |  |  |
| 8. | Dump site pumper driver/operator or jet siphon pumper driver/operator: Monitor the level of water in each tank and adjust the siphon lines accordingly. |  |  |