Skill 13-3

Evaluation Checklist

Objective 8:	Operate at a portable water tank dump site as part of a water shuttle operation. [NFPA® 1002, 10.2.2, 10.2.3]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will position a mobile water supply apparatus at a water shuttle dump site. Students must work as part of a crew for this skill sheet. A spotter is required for this skill. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Appropriate PPE, including high visibility traffic vest for spotter - Area to allow easy access for water tenders (preferably no backing) - Pumping apparatus - Hard suction with strainer | - Mobile water supply apparatus - Salvage cover - Portable water tank |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 8:	Operate at a portable water tank dump site as part of a water shuttle operation. [NFPA® 1002, 10.2.2, 10.2.3]

CAUTION: The apparatus should be fully emptied before allowing it to proceed to the fill site. A partially loaded tanker will cause water movement that can affect the stability of the apparatus.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| Selecting Dump Site Location |  |  |  |
| 1. | Position in the appropriate location. a. Close proximity to incident scene b. Multiple access points c. Consideration given to fire growth and possible exposure d. Allows other apparatus access to the fire scene CAUTION: If working at night, ensure the fill site has adequate lighting, but not blinding to drivers. |  |  |
| Deploying Portable Tanks |  |  |  |
| 2. | Crew and mobile water supply apparatus driver/operator: Deploy a salvage cover on the ground where the dump tank will be set-up. |  |  |
| 3. | Crew and mobile water supply apparatus driver/operator: Remove the dump tank from the apparatus and follow the manufacturer’s instructions for deploying the tank. a. Ensure a drain for the tank is in the downhill side of the tank so that the tank may be easily drained at the end of the incident. |  |  |
| 4. | Crew and mobile water supply apparatus driver/operator: Connect the hard suction strainer to the hard suction hose that is connected to the dump site pumper and place the hose in the tank. |  |  |
| Operating at Site |  |  |  |
| 5. | Spotter: Signal the mobile water supply apparatus into position. CAUTION: If apparatus have to back-up, ensure that a common set of hand or radio signals are established and a spotter is always present. Immediately stop backing the apparatus if you lose sight of the ground guide spotter. |  |  |
| 6. | Mobile water supply apparatus driver/operator: Come to a complete stop on command, set the parking brake and place the apparatus into neutral. |  |  |
| 7. | Dump site crew: Operate the dump/discharge valve(s) to allow the tank to be dumped into the portable water tank. |  |  |
| 8. | Dump site crew: Once the mobile water supply apparatus is empty, close the valve(s) and signal the driver/operator to proceed to the fill site. |  |  |
| 9. | Dump site crew: Signal the next mobile water supply apparatus to pull into dumping position after checking that the first mobile water supply apparatus, hoselines, and personnel are clear. |  |  |