Skill 3-1

Evaluation Checklist

Objective 15:	Start, idle, and shut down a fire service apparatus. [NFPA® 1002, 4.3.1]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will start, idle, and shut down a fire service apparatus. Remind students to always follow manufacturer’s recommendations and department standard operating procedures (SOPs).

Resources

| - Manufacturer’s operator’s manual - Fire service apparatus - Appropriate PPE including hearing protection |  |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 15:	Start, idle, and shut down a fire service apparatus. [NFPA® 1002, 4.3.1]

NOTE: Ensure vehicle is properly prepared and that all personnel are clear before performing this skill.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Disconnect all external electrical cords, air hoses, or exhaust system hoses from the apparatus. |  |  |
| 2. | Mount the apparatus, using appropriate steps and handrails. a. Don safety restraints. b. Ensure that all riders are wearing safety restraints and hearing protection |  |  |
| 3. | Turn on the vehicle battery(ies). CAUTION: Never operate the battery switch while the engine is running. NOTE: Depending on the location of the battery switch, you may choose to operate this switch before entering the cab or immediately upon sitting in the driver's seat. |  |  |
| 4. | Check that the parking brake is set. |  |  |
| 5. | Manual transmissions only: Place the transmission in neutral. |  |  |
| 6. | Turn on the ignition switches; wait until the vehicle computers and systems go through normal start-up. |  |  |
| 7. | Operate the starter control, following manufacturer’s recommendations. |  |  |
| 8. | Observe the apparatus gauges. a. Shut down the engine immediately if oil pressure gauge does not fall within manufacturer’s recommendations within 5 to 10 seconds. b. Check ammeter and/or voltmeter to make sure that electrical system is operating/charging properly. c. Check air pressure gauge to ensure that adequate pressure is built up to release parking brake and operate service brake. |  |  |
| 9. | Shut off the engine following manufacturer’s procedures. CAUTION: Never operate the battery switch while the engine is running. Do not attempt to shut down the engine with the battery switch. |  |  |
| 10. | Turn the battery switch to the OFF position. |  |  |
| 11. | Reconnect external electrical cords, air hoses, or exhaust system hoses to the apparatus. |  |  |