Skill 2-1

Evaluation Checklist

Objective 7:	Clean the interior and wash and wax the exterior of a fire department apparatus.

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will wash, wax, and dry a fire service apparatus, including the exterior, glass, tires, and interior of a fire service apparatus. This skill may be done individually or with several other firefighters. Remind students to always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures.

Resources

| - Fire service apparatus - Manufacturer’s operators manual - Wheel chocks - Garden hose, nozzle, and water supply source - Squeegee - Bucket(s) | - Manufacturer-approved cleaning agents: - Automotive wax or protective finish - Grease/tar removing solvent - Glass cleaner - Interior cleaning agent - Interior protective coating - Clean soft cloths and chamois - Non-abrasive scrub brushes |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 7:	Clean the interior and wash and wax the exterior of a fire department apparatus.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Preparation |  |  |
| 1. | Park the apparatus in an appropriate location; starting, driving, and braking according to local SOPs. |  |  |
| 2. | Chock the apparatus wheels. |  |  |
| 3. | Gather equipment and set up work area. |  |  |
|  | Interior Cleaning |  |  |
| 4. | Sweep or vacuum large, loose dirt particles. |  |  |
| 5. | Clean the interior surfaces, using warm soapy water or other manufacturer-approved products. NOTE: Take care when using any liquids around the vehicle’s electrical system. Use of a water hose to flush out the interior floor area can result in electrical/electronic failures. |  |  |
| 6. | Clean any debris from compartments, hinges, slide tracks, and seals. Lubricate according to manufacturer’s specifications. |  |  |
| 7. | Clean interior glass using warm soapy water or glass cleaner in conjunction with paper towels or clean cloth rags. NOTE: Do not use anything abrasive on window glass. |  |  |
|  | Washing the Exterior |  |  |
| 8. | Apply water to the apparatus from top to bottom using a garden hose without a nozzle. Rinse as much loose dirt as possible from the vehicle before applying cleaning agent. NOTE: Do not wash with extremely hot water or while the surface of the vehicle is hot. |  |  |
| 9. | Wash the apparatus from top to bottom using appropriate automotive cleaning products. |  |  |
| 10. | Dry the apparatus exterior completely using a clean chamois or rubber squeegee. |  |  |
| 11. | Clean exterior glass using warm soapy water or glass cleaner in conjunction with paper towels or clean cloth rags. NOTE: Do not use anything abrasive on window glass. |  |  |
|  | Wax or Protective Finish |  |  |
| 12. | Apply wax or protective finish following manufacturer’s recommendations. |  |  |