Skill 2-6

Evaluation Checklist

Objective 12:	Charge an apparatus battery. [NFPA® 1002, 4.2.1]

| Student Name: |  | Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will charge an apparatus battery. Remind students to always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures.

Resources

| - Fire service apparatus - Manufacturer’s operators manual • Wheel chocks • Appropriate PPE: eye protection, gloves and station wear | • Vehicle battery • Battery charger • Apparatus maintenance logs/forms • Writing utensil |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skills Evaluation Checklist

Objective 12:	Charge an apparatus battery. [NFPA® 1002, 4.2.1]

WARNING: Batteries produce explosive hydrogen gas when being charged. Chargers must be used correctly to prevent accidents, injury, and even fatalities. Ventilate the area when servicing. Wear safety goggles and acid-resistant gloves. Always follow apparatus and battery charger manufacturer’s instructions.

NOTE:  The following are general steps for charging an apparatus battery. Always follow manufacturer specific instructions.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Read manufacturer’s operators manual for the battery charger. Ensure you are familiar with information on the specific vehicle and battery used. |  |  |
| 2. | Don appropriate PPE. |  |  |
| 3. | Ensure the charging area is appropriately set up. Change batteries in a designated area that is well ventilated. |  |  |
| 4. | Make sure that the battery and ignition switch(es) are in their OFF positions. |  |  |
| 5. | Clean the battery terminals prior to charging. |  |  |
| 6. | Identify the polarity and voltage of the charging system of the battery to be charged (positive or negative ground). |  |  |
| 7. | Connect the charging cables in accordance with manufacturer’s instructions. For example, the positive cable to the positive post of the battery and the negative cable to the stand-alone negative post or vehicle frame. NOTE: The positive charger cable is usually marked “Pos or +” and is often colored red. The negative charger cable is usually marked “Neg or −” and is colored black. |  |  |
| 8. | Follow manufacturer instructions on connecting the battery charger to a power source. |  |  |
| 9. | Follow manufacturer instructions for charging the battery. |  |  |
| 10. | Follow manufacturer instructions for disconnecting the battery charger. |  |  |