Fire Officer I

Activity 16-1 NFA Fire Flow Calculations

Instructions:

Small Group Activity – each group is assigned one scenario to calculate.

A group spokesperson comes to the board and goes over the group’s calculation.

Each group answers these three questions:

- What is the Needed Fire Flow (NFF) requirement?

- Are there any exposures and how much water is added to the NFF?

- What size lines would be required for both supply and attack?

LxW/3 = GPM 100% involvement-All Calculations Start With This Number, Needed Fire Flow (NFF) is the amount needed based on the percentage of fire involvement. Thus the 100% number divided by the percentage of involvement will give you the Needed Fire Flow. Exposures within 50 feet are figured at 25% of the 100% number (25% for each exposure). Floors above the fire(up to 4 floors) are considered exposures(25% for each floor).

Scenario A: A single story wood frame residential structure is 30 x 40 feet and fully involved in fire. A detached garage is within 20 feet of the rear of the structure.

30 x 40 = 1200  1200/3 = 400

100% = 400 gpm

One exposure  400/.25 = 100 gpm

Total needed Fire Flow 400(structure) + 100(exposure) = 500 gpm

Scenario B: A three story commercial building is 80 x 80 feet and 50% involved on the first floor.

Scenario C: There is a fire on the stovetop in the restaurant inside Kmart – you estimate the building to be 1% involved. Kmart measures 200 x 250 feet. There are no immediate exposures.

Scenario D: A repair garage is 75 x 50 feet and is 25% involved in fire in the repair bay. A very large pile of used tires are stacked up 10 feet high on the property line which is 60 feet from the garage.