Strategy and Tactics Considerations

S-Size-up, the mental evaluation of the situation

- FPODP

  - Facts-What do I see

  - Probabilities-Life safety factor, Is there a probable victim

  - Own situation-Resources, am I alone or do I have immediate help

  - Decision-Offensive or defensive strategy

  - Plan of action- Deployment of tactical resources, who does what

A-Apparatus placement

- Three sides seen before stepping out of the apparatus

- Out of the hot zone

  - Heat

  - Collapse

  - HAZMAT

  - Uphill/upwind

  - Power lines

W-Water supply, you must be able to support your operation

- Who (which company is assigned)

- What (what type of supply)

- Where (where is the source)

C-Command

- Who (command option or pass command)

- What (what is the command name)

- Where (where is the CP located)

S-Strategy-Offensive or defensive

- Sets the general direction of the operation

S-Safety

- RIC

- Fireground accountability tracking system

- Utility control

R-Rescue is the first tactical priority

- Assigned who and where

- Primary search

- Secondary search

- All clear

E-Exposures

- Internal (attic/cockloft, floor above, other rooms, egress. stairwells etc.)

- External (all four sides, downwind, adjacent occupancies, common attics)

- Auto-exposure

C-Confinement

- Hose line placement

  - Protect uninvolved areas, occupants and means of egress

E-Extinguishment

- Line and nozzle size to satisfy fire flow

O-Overhaul

- Who and where

- Prioritize based on construction type I.E. balloon type vs. platform construction

V-Ventilation

- Vent early and aggressively

- Location and type of ventilation

  - Vertical

  - Horizontal

  - PPV

  - Offensive heat hole

  - Defensive trench cut

  - Must be coordinated with attack

S-Salvage

- Identify salvage priorities

- Contact RP

S-Support-Call for additional resources ahead of time. Resources to consider:

- Greater alarms

- Personnel (safety officer, PIO, Tech specialists)

- Special equipment (Air truck, HAZMAT team HRT)