EVIDENCE COLLECTION

- Demonstrate evidence collection of a solid/liquid functioning as the clean technician.

REFERENCE:	Sampling protocol developed by the FBI.

CONDITION:	Given crime scene has been rendered safe to enter and a written sampling plan has been approved. Utilize an evidence collection kit, evidence collection flow sheet, and 2-member sampling team. Unknown solid/liquid that is uncontained or located in opened containers. PPE shall be a minimum of Splash Suit, SCBA. *PPE donned before skill begins.

COMPETENCE:	Skills for clean technician.

- NEVER collects samples.

- Seals and labels primary control container in clean evidence collection bag in an area free of contamination.

- Coordinates with dirty tech to establish a clean work area using a clean disposable sheet or ground pad.

- Handles clean equipment only.

- Transfer clean evidence containers and evidence collection sampling tools to dirty tech in area separate from clean equipment.

- Receives sealed, labeled primary container in clean evidence collection bag or container.

- NEVER touches samples unless they are sealed.

- Ensures each primary and secondary container is sealed and labeled with a sample number, date, case number, initials of sampler, and where collected.

- Documents evidence collection and field screening on log

- Maintains control over collected evidence.

- Follows “chain of custody” procedures when delivering evidence to evidence custodian.

TIME:	15:00 Minutes

- Demonstrate evidence collection of a solid/liquid functioning as the dirty technician.

REFERENCE:	Sampling protocol developed by the FBI.

CONDITION:	Given crime scene has been rendered safe to enter and a written sampling plan has been approved. Utilize an evidence collection kit, evidence collection flow sheet, and 2-member sampling team. Unknown solid/liquid that is uncontained or located in opened containers. PPE shall be a minimum of Splash Suit, SCBA. *PPE donned before skill begins.

COMPETENCE:	Skills for dirty technician.

- Performs field screening prior to collection.

  - Ensures materials are cleared of Improvised Explosive Devices by Bomb Technician Units.

  - Checks for crystallized materials around caps, and threads of containers.

  - Does not open any unopened containers unless under the advice of the HazMat Officer.

  - Checks for Alpha, Beta, and Gamma radiation.

  - Checks vapor space for corrosive using pH paper.

  - Checks for Oxygen levels.

  - Checks for Flammability.

  - Checks for Toxicity.

  - Utilizes Colorimetric Tubes if a specific material is suspected.

- Utilizes proper evidence collection sampling tool.

  - If sealed, opened and handled from the handle end only.

- Collects unconfined solid/liquid or solid/liquid from open container.

- Uses “clear” compatible containers.

  - Suspected biological samples collected in sterile containers.

  - Suspected chemical samples collected in either sterile or certified clean containers.

- Returns lid to container and seals container with Parafilm.

- Labels primary container confirming sample number.

- Coordinating with clean technician, places primary container in secondary container without touching it.

- Changes gloves in between samples and/or sampling spots in preparation for next sample.

TIME:	15:00 Minutes