Fire Inspector 1-Assignment #5

Utah Fire and Rescue Academy

September 2022

Situation:

You have been assigned to do the plan review on a building that was formally a restaurant. It has been purchased by a toy manufacturer. The existing building is fully fire sprinklered with a light hazard system and all doors are three feet wide. This company manufactures wooden toys. It is their intent to remove the kitchen and restaurant seating, to remove all of the ceilings and to set up a full wood shop. The building will be heated except for the storage room.  Refer to drawing assignment #5.  Plans are not to scale.

Assignment:

Answer the following questions. Include the 2018 IFC (International Fire Code) References for each question.

- What was the original occupancy?

- What is the new occupancy?

- What is the occupant load factor?

- What is the new occupant load?

- What are the interior dimensions of this bldg. (Scale 1/8” = 1’-0”) plan is not to scale

- What is the required exit width?

- Number of exits required?

- What is the travel distance to an exit?

- Can the kitchen door be used as an exit?

- Minimum distance between exits in the shop area.

- Maximum IB flammable liquid allowed in an appropriate storage cabinet?

- Identify applicable codes and standards?

- Are exit signs required? If so, where?

- Is panic hardware required?

- Is exit (egress) illumination required?

- New stairs have been installed outside but the rise of each stair varies by 3/8”. Is this acceptable?

- A new ramp was built with the stairs. The ramp is 48” wide and constructed of 2 x 4’s and plywood. The rise is 28” with a handrail on one side. There is one change of direction with a landing measuring 48” x 48”. Is the ramp acceptable?

- What are the dimensions of the word “exit” in an exit sign?

Requirements:

- Answer the questions and give the IFC code references. Use a word processing program to type your answers.

- Submit the assignment electronically (e-mail) to the lead instructor.

State Skills 5,6,7,8,16,17