Inspector I – Assignment #6

Utah Fire and Rescue Academy

September 2022

Situation:

Fire-flow tests are made to determine available and required fire flow for fire suppression. Knowing how to collect the fire flow information, mathematically determine water flow, graphically determine available flow, and determine required flow per code for a particular building or project is an essential skill for the fire inspector.  Using the IFSTA Manual and 2018 IFC (International Fire Code).

Assignment:

Answer the following questions;

Part 1 – Conduct a Fire Flow Test

- What tools are required to perform a fire hydrant flow test?  					____________

_________

_________

Part 2

For this portion of the activity, you will be determining fire flow. Using the information given in each problem below and the fire-flow formula, determine the fire flow for each hydrant outlet.

Please round your answer to two (2) decimals.

Fire-Flow formula

GPM = (29.83) x C x d2 x √P

1.	Given:

Flow pressure = 20 psi

Coefficient of rounded outlet = 0.9

Diameter = 2 7/16 or 2.44 inches

What is the fire flow for this hydrant outlet?

2.	Given:

Flow pressure = 25 psi

Coefficient of square outlet = 0.8

Diameter = 2 15/16 or 2.94 inches

What is the fire flow for this hydrant outlet?

3.	Given:

Flow pressure = 40 psi

Coefficient of projecting outlet = 0.7

Diameter = 3 1/8 or 3.13 inches

What is the fire flow for this hydrant outlet?

4.	Given:

Flow pressure = 28 psi

Coefficient of square outlet = 0.8

Diameter = 2 5/16 or 2.3 inches

What is the fire flow for this hydrant outlet?

5.	Given:

Flow pressure = 32 psi

Coefficient of projecting outlet = 0.7

Diameter = 1 5/16 or 1.3 inches

What is the fire flow for this hydrant outlet?

Part 3 – Using the water supply graph, graph the following scenarios down to determine the available water supply at 20 psi.

- Static:_40___  	Residual:_30___ 	Flow:_550__	Flow at 20 psi?________

- Static:_135__  	Residual:_100__	Flow:_1000_	Flow at 20 psi?________

- Static:_70___  	Residual:_35___ 	Flow:_1340_	Flow at 20 psi?________

- Static:_105__	Residual:_52_    	Flow:_1200_	Flow at 20 psi?________

- Static: _128  _	Residual:_80___ 	Flow:_880__	Flow at 20 psi?________

Part 4 – Using IFC Appendix B & C, determine the required fire flow following the buildings?

Given a typical mercantile store, type 2B construction, approximately 8,800 square feet?

Fire Flow ? ______ Number of Hydrants ?______

Given a typical wood shop, type 5 construction, approximately 18,500 square feet?

Fire Flow ?______ Number of Hydrants ?______

Given a typical hospital, type 1A construction, approximately 154,500 square feet?

Fire Flow ?______ Number of Hydrants ?_______

Requirements:

- Answer all questions.

- Answer questions using word processing software.

- Submit Assignment #6 electronically (email to lead instructor).

State Skill #19